Intel® Media Streaming Library for ST 2110 User Guide

Intel Corporation, Document Version: 0.1

LEGAL DISCLAIMER

INFORMATION IN THIS DOCUMENT IS PROVIDED IN CONNECTION WITH INTEL PRODUCTS. NO

LICENSE, EXPRESS OR IMPLIED, BY ESTOPPEL OR OTHERWISE, TO ANY INTELLECTUAL

PROPERTY RIGHTS IS GRANTED BY THIS DOCUMENT.  EXCEPT AS PROVIDED IN INTEL'S

TERMS AND CONDITIONS OF SALE FOR SUCH PRODUCTS, INTEL ASSUMES NO LIABILITY

WHATSOEVER AND INTEL DISCLAIMS ANY EXPRESS OR IMPLIED WARRANTY, RELATING TO

SALE AND/OR USE OF INTEL PRODUCTS INCLUDING LIABILITY OR WARRANTIES RELATING TO FITNESS FOR A PARTICULAR PURPOSE, MERCHANTABILITY, OR INFRINGEMENT OF ANY PATENT, COPYRIGHT OR OTHER INTELLECTUAL PROPERTY RIGHT.

UNLESS OTHERWISE AGREED IN WRITING BY INTEL, THE INTEL PRODUCTS ARE NOT

DESIGNED NOR INTENDED FOR ANY APPLICATION IN WHICH THE FAILURE OF THE INTEL

PRODUCT COULD CREATE A SITUATION WHERE PERSONAL INJURY OR DEATH MAY OCCUR.

Intel may make changes to specifications and product descriptions at any time, without notice. Designers must not rely on the absence or characteristics of any features or instructions marked "reserved" or "undefined." Intel reserves these for future definition and shall have no responsibility whatsoever for conflicts or incompatibilities arising from future changes to them. The information here is subject to change without notice. Do not finalize a design with this information.

The products described in this document may contain design defects or errors known as errata which may cause the product to deviate from published specifications. Current characterized errata are available on request.

Contact your local Intel sales office or your distributor to obtain the latest specifications and before placing your product order.

Copies of documents which have an order number and are referenced in this document, or other Intel literature, may be obtained by calling 1-800-548-4725, or by visiting Intel's Web Site.

Intel and the Intel logo are trademarks or registered trademarks of Intel Corporation or its subsidiaries in the United States and other countries.

*Other names and brands may be claimed as the property of others.

Copyright © 2020, Intel Corporation. All Rights reserved.

Table of Contents

Overview	6

Acronyms and Abbreviations	6

Requirements	7

Operation concept	7

Flow diagram	10

Key parameters	11

API Diagram	12

StDevice API group	13

StInitLib – deprecated, to be removed after alpha	13

StCreateDevice	13

StStartDevice	14

StDestroyDevice	14

StPtpSetClockSource	15

StPtpGetClockSource	15

StPtpSetParam	16

StPtpGetParam	16

St21 API group	17

St21CreateSession	17

St21BindIpAdrr	19

St21ListenSession	20

St21JoinSession	20

St21DropSession	21

St21GetFormat	21

St21SetParam	22

St21GetParam	22

St21 Producer API group	24

St21RegisterProducer	24

St21ProducerStartFrame	26

St21ProducerUpdate	26

St21ProducerStop	27

St21 Consumer API group	27

St21RegisterConsumer	27

St21ConsumerStartFrame	30

St21ConsumerUpdate	30

St21ConsumerStop	31

Compilation and building	31

Installation	31

Run	32

Sample execution parameters	32

Change execution parameters	32

# Overview

The Intel® Media Streaming Library for ST 2110 is an easy to implement set of C language APIs that will allow for the development of applications which perform lossless high-speed video transmission over IP networks as specified by the SMPTE 2110 family of standards. SMPTE (Society of Motion Picture and Television Engineers) is a well-established organization that has driven this next generation wide industry adapted set of digital video transmission standards. SMPTE ST 2110 specifies the carriage, synchronization, and description of separate elementary essences for video, audio, and data over IP networks.  A wide range of software and hardware products in the video Industry use this standard for real-time production, playout, and other professional media applications.

The library will allow a developer to focus on the processing of video frames and lines while utilizing the library to manage the fine details of forming IP packets with RTP encapsulation and decapsulation of efficiently packed pixel data, along with nanosecond precise PTP timing.  Up to 100GBps traffic per port of video, audio, and ancillary essences as defined in the ST2110 standard can be generated using this library to develop transmit and receive applications experiencing very low latency (e.g. 1 frame end to end).

DPDK (Data Plane Development Kit) is a very popular software solution for the problem of handling high speed transmission of data which bypasses Linux kernel bottlenecks including interrupts and memory copies. A key advantage of the Intel® Media Streaming Library for ST 2110 is that it uses DPDK efficiently and provides high performance processing of packets as they are sent and received from network interface cards.

A wide range of software applications can be developed taking advantage of this library's support for multiple sessions, diverse pixels formats, frame sizes and rates, interlacing, multicasting, and network speeds.  In addition to continued optimization and support for even higher network speeds, future versions of this library will allow for containerized applications running in virtual environments, more advanced packet pacing, and supporting stream redundancy for improved transmission reliability.

## Acronyms and Abbreviations

| BDF | Bus:Device:Function – system ID of a PCI device |
| --- | --- |
| DPDK | Data Plane Development Kit |
| Frame | Refers to video frame |
| Frame Epoch | Time of the frame start equal to the time in which the frame would start under assumption that the 1st frame was send at the UPC epoch (so that the is integer number of complete frames since then) |
| Frame rate | Video frames per second rate |
| KM | Kernel Mode |
| KMD | Kernel Mode Driver |
| MTU | Maximal Transfer Unit of a packet allowed on network, for IP packet on Ethernet this is 1500. |
| Packet | Packet (byte frame) on network |
| PTP | Precision Time Protocol |
| RTP | Real Time Protocol |
| SDP | Session Description Protocol – Sender metadata that helps to identify RTP session format and essence |
| SMPTE | Society of Motion Pictures and Television Engineers |
| St21 Session | RTP Session of Raw Video complying to RFC 4175 |
| St30 Session | RTP Session of PCM Audio |
| St31 Session | RTP Session of AES3 Audio |
| St40 Session | RTP session with Ancillary data |
| UDP | User Datagram Protocol, Layer 4 protocol on the top of IPv4 or IPv6 protocol |
| UM | User Mode |
| UMD | User Mode Driver |
| VRX | Virtual Receive Buffer |
| RVRTP | Raw Video Real Time Protocol |
| UIO | Userspace Input/Output |
| IGB | Intel GigaBit Ethernet Driver |

## Architecture overview

### Sender – receiver transmission stack.

Video Multichannel Producer Application

Customer SW that calls the Intel Media Streaming library for ST 2110 and in case of video producer provides callback interface to drain the video buffers,

Video Multichannel Consumer Application

In case of video consumer provides callback interface to get notification about received frames or frame slices. The data interface to the library is through the video buffers that contain CbY0CrY1 10bit packet color (5 bytes are used to hold CbY0CrY1 color that are for 2 consecutive pixels).

### StDevice components for Transmitter and Receiver.

StDevice implements fundamental library structure in each direction, on which Video, Audio and Ancillary sessions are instantiated. Usually a single device is created for a pair of NIC ethernet ports, so that naturally can provide the redundant path capability – streams addressed for different destinations but having the same timing and payload, so that the same video essence travels a different path in the network. In case of sender device, it initializes the pool of timeslots through dividing the frame time into slices equal to the default packet pacing distance – for example in case of 50fps rate the time slice size is 4.44us for 1080p traffic. Then each slice can send a number of streams, in case of 10Gbps NIC port within 4.44us 4 packets of default video packet size of 1268 can be sent.

St21Session component is responsible for raw video session setup and management, for direction of transmitter it registers within device for timeslots that guarantee smooth transition of the video. It is possible the base rate of the device is not compatible with the rate and resolution of the session, please refer to section 6.1.3.1. Each session has callback registered from the sender or receiver application. The session component is responsible for issuing these callbacks to drain the video buffer for the transmitter side and notify about frame completion on the receiver side. Each video session on transmitter side has assigned scheduler ring. The St21Session API is not intended for multi-process communication. The API will work within the same process as library is instanced. To achieve multi-process communication, several approaches are possible, one is to add shared memory to exchange data efficient with other processes and implement inter process notifications on the top of the session callbacks.

St30Session(not implemented in prototype release) component is responsible for audio session setup and management (PCM AES67 or AES3), for direction of transmitter it registers within device for time budget within multichannel audio packets that guarantee smooth transition of the audio (which is tiny compared to video). Audio packet per each session is send every 1ms and usually from 2 to 8 channels form a single, bigger audio packet to save the bandwidth. All audio channels are sent via socket API, which then in the library, use one of remaining send device rings (since video cannot occupy more than 90% of the NIC bandwidth).

St40Session (not implemented in prototype release)

Ancillary data

RFC4175 Raw Video RTP encapsulation component (also called rvrtp rings) performs RTP encapsulation and puts packets to the scheduler ring. It is also responsible for aligning the 1st packet of the frame to the Epoch + Troffset time. The alignment algorithm calculates the time to the next frame beginning and waits in draining the video buffer through missing time slices (Tprs), therefore for example of 50fps frame rate of 1080p session, a step of delay is Trps time of 4.44us, for 25fps is 8.88us.

On the receiver side, RFC4175 Raw Video RTP component is responsible for decapsulation of RTP packets (and verification). Each received stream has assigned one of available RX queues, so that multiple threads can easily parallelize reception of the packets of different sessions.

### Flow diagram

## Typical TX application setup for unicast IP destination

* - not implemented in the prototype release

## Typical TX application setup for multicast IP destination

* - not implemented in the prototype release

## Typical RX application setup

* - not implemented in the prototype release

# API Reference

Figure 3. ST Library API Structure diagram

## 2. StDevice API group:

### 2.1 StSetParam

Notation:

st_status_t

StGetParam(
  st_param_t prm,
  st_param_val_t val);

typedef enum st_param

{

ST21_FRM_FIX_MODE = 10,		//configurable, by default enabled for ST21_FRM_FIX_PREV_N_2022_7

ST21_FRM_2022_7_MODE = 11,	//configurable, by default enabled if 2nd port is configured

ST21_TPRS = 20,			//value shall be in nanoseconds, read-only

ST21_TR_OFFSET = 21, 		//value shall be in nanoseconds, read-only

ST21_FRM_TIME = 22,		//value shall be in nanoseconds, read-only

ST21_PKT_TIME = 23,	 	//value shall be in nanoseconds, read-only

ST21_PIX_GRP_SZ = 30,		// read-only

ST_BUILD_ID = 40,		// read-only

ST_LIB_VERSION = 41,		// read-only

ST_PTP_DROP_TIME = 100,

ST_PTP_THRESHOLD = 101,

ST_SOURCE_IP = 150,

ST_DESTINATION_IP = 151,

ST_EBU_TEST = 152,

ST_SN_COUNT = 153,

ST_TX_ONLY = 154,

ST_RX_ONLY = 155,

ST_MAC = 156,

ST_OUT_PORT = 157,

ST_IN_PORT = 158,

ST_FMT_INDEX = 159,

ST_DPDK_PARAMS = 160,

} st_param_t;

typedef union

{

uint32_t valueU32;

uint64_t valueU64;

char 	*strPtr;

} st_param_val_t;

Description:

The function shall be called in order to set library specific parameter like for example: static destination MAC address, enable/disable of printing EBU statistics, etc.

Remarks:

Supported since first SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### 2.2 StGetParam

Notation:

st_status_t

StGetParam(
  st_param_t prm,
  st_param_val_t *val);

typedef enum st_param

{

ST21_FRM_FIX_MODE = 10,		//configurable, by default enabled for ST21_FRM_FIX_PREV_N_2022_7

ST21_FRM_2022_7_MODE = 11,	//configurable, by default enabled if 2nd port is configured

ST21_TPRS = 20,			//value shall be in nanoseconds, read-only

ST21_TR_OFFSET = 21,		//value shall be in nanoseconds, read-only

ST21_FRM_TIME = 22,		//value shall be in nanoseconds, read-only

ST21_PKT_TIME = 23,		//value shall be in nanoseconds, read-only

ST21_PIX_GRP_SZ = 30,		// read-only

ST_BUILD_ID = 40,		// read-only

ST_LIB_VERSION = 41,		// read-only

ST_PTP_DROP_TIME = 100,

ST_PTP_THRESHOLD = 101,

ST_SOURCE_IP = 150,

ST_DESTINATION_IP = 151,

ST_EBU_TEST = 152,

ST_SN_COUNT = 153,

ST_TX_ONLY = 154,

ST_RX_ONLY = 155,

ST_MAC = 156,

ST_OUT_PORT = 157,

ST_IN_PORT = 158,

ST_FMT_INDEX = 159,

ST_DPDK_PARAMS = 160,

} st_param_t;

typedef union

{

uint32_t valueU32;

uint64_t valueU64;

char 	*strPtr;

} st_param_val_t;

Description:

The function shall be called in order to retrieve library specific parameter value like for example: static destination MAC address, enable/disable of printing EBU statistics, etc.

Remarks:

Supported since the first SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### 2.3 StCreateDevice

Notation:

st_status_t

StCreateDevice(

st_device_t *inDev,    //IN structure with device parameters

const char *port1Bdf,  //IN BDF of primary port PCI device

const char *port2Bdf,  //IN BDF of secondary port PCI device

st_device_t **outDev); //OUT created device object w/ fields updated per link capabilities

struct st_device

{

st_version_t    ver;//IN requested version of the API

st_dev_type_t   type;//IN requested device type, one of producer or consumer

st_pacer_type_t pacerType;//IN requested pacing type

st_exact_rate_t exactRate;//IN requested exact base frame rate

uint32_t        rateGbps;//OUT, determined by a NIC port rate in Gbps, 10-100 are expected values

uint16_t        port[2]; //OUT, DPDK port IDs as selected with the BDFs

uint16_t        mtu;//if > 1500 requested MTU, updated with value possible on the links

uint16_t        maxSt21Sessions;//IN arbitrary maximal number of ST21 video sessions

uint16_t        maxSt31Sessions;//IN arbitrary maximal number of ST21 audio sessions

uint16_t        maxSt40Sessions;//IN arbitrary maximal number of ST40 ancillary sessions

};

Description:

The function is called to initialize the library and create a single device for either a transmit or receive operation. By creating both kind of devices the library is configured to do both operations simultaneously. Only a single device per each direction can be created at a time for a pair of ports.

The device is created on ports (specified by port1Bdf and port2Bdf) that are represented by a PCIe Bus:Device.Function string, for example 0000:af:00.1  - NIC device 0x0 function 0x1 (usually 2nd port) seating on a PCIe bus 0xaf. 2 ports are required to setup a redundant path streaming. i.e. the 2nd specified port acts as a video streaming redundancy mirror (with different IP and VLAN addresses compared to 1st path stream).

The meaning of the other function parameters is as follows:

- inDev structure can be used to force arbitrary number of sessions to a lower value than is available on the NIC port.  It also selects the following options for the device:

  - type selects device type either  ST_DEV_TYPE_PRODUCER or ST_DEV_TYPE_CONSUMER

  - pacerType selects the sender type,either  ST_2110_21_TPW, ST_2110_21_TPNL, or ST_2110_21_TPN

  - exactRate selects the video frame rate either ST_DEV_RATE_29_97, ST_DEV_RATE_25_00, ST_DEV_RATE_59_95, or ST_DEV_RATE_50_00

  - maxSt21Sessions selects maximal number of ST21 uncompressed video sessions allowed to be created on the device

  - maxSt30Sessions selects maximal number of ST30 uncompressed audio sessions allowed to be created on the device

  - maxSt40Sessions selects maximal number of ST40 ancillary sessions allowed to be created on the device

- outDev pointer is used to return the created device handle that can be used in the following API calls upon successful completion.

Remarks:

Supported since first SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

ST_OUT_OF_MEMORY

### 2.4 StStartDevice

Notation:

st_status_t

StStartDevice(st_device_t *dev)

Description:

The function shall be called to start device operation for transmit or receive operation.

Remarks:

Supported since the first SW release.

Return codes and errors:

ST_OK

ST_GENERAL_ERROR

### 2.5 StDestroyDevice

Notation:

st_status_t

StDestroyDevice(st_device_t *dev)

Description:

The function is called to free allocated device resources before gracefully closing the application.

Remarks:

Supported since prototype SW release.

Return codes and errors:

ST_OK

ST_GENERAL_ERROR

### 2.6 StPtpSetClockSource

Notation:

st_status_t

StPtpSetClockSource(

st_ptp_clock_id_t *priClock, //IN pointer to the clock ID of primary PTP Grandmaster

st_ptp_clock_id_t *bkpClock);//IN, optionally pointer to the clock ID of backup PTP Grandmaster

Description:

The function is called to enable accepting PTP frames from the specific grandmaster clock source. The primary clock source must be specified, while the backup clock source is optional.

Remarks:

Supported since prototype SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### 2.7 StPtpGetClockSource

Notation:

st_status_t

StPtpGetClockSource(

st_ptp_clock_id_t *currClock);//OUT, pointer to the clock ID that is being currently used as Grandmaster

Description:

The function is called to get the active PTP grandmaster ID.

Remarks:

Supported since prototype SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### 2.8 StPtpSetParam

Notation:

st_status_t

StPtpSetParam(

st_param_t prm, //IN param ID

uint64_t val);  //IN value to be set

Description:

The function is called to set PTP specific parameters. Supported parameters are:

ST_PTP_DROP_TIME  - Drop time after which the backup PTP clock is used instead of primary PTP. Also once the primary clock is back (online?) then it needs to be stable for at least as long as the interval to switch back. In that case val contains threshold (in nanoseconds), the value above which the backup PTP Grandmaster is used as the active Grandmaster.

ST_PTP_THRESHOLD - PTP adjustment threshold time above which the Ptp (What is the Ptp timer??) timer gets PTP Grandmaster clock unconditionally as is (since the difference is too large). Otherwise, half of the difference is used to adjust the local PTP clock. If the threhold time is set to 0 then the PTP Grandmaster gets as is (default behavior). Then the value of val is then threshold (in nanoseconds), is the value above which the received value of PTP Grandmaster is used as the local PTP clock s frames from the specific clock source Grandmasters. The primary clock source must be specified while the backup clock source is optional.

Remarks:

Supported since prototype SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### 2.9 StPtpGetParam

Notation:

st_status_t

StPtpSetParam(

st_param_t prm, //IN param ID

uint64_t *val); //OUT value to be get

Description:

The function is called to retrieve PTP specific parameters. Supported parameters are:

ST_PTP_DROP_TIME  - Drop time after which the backup PtP clock is used instead of primary Ptp. Also aonce the primary clock is back then it needs to be stable as least as long this interval to switch back. In that case the location pointed by val is updated with the threshold (in nanoseconds), threshold above which the backup PTP Grandmaster is used as the active Grandmaster.

ST_PTP_THRESHOLD - PTP adjustment threshold time above which the Ptp timer gets PTP Grandmaster clock unconditionally as is (since the difference is too large). Otherwise, half of the difference is used to adjust the local PTP clock. In that case the location pointed by val is then updated with the value of threshold (in nanoseconds) that is set in the library.

Remarks:

Supported since prototype SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

# 3. St21 API group

### 3.1 St21CreateSession

Notation:

st_status_t

St21CreateSession(

st_device_t *dev,       // IN device on which session is created

st21_session_t *inSn,   // IN structure of session params

st21_format_t *fmt,     // IN session packet's format

st21_session_t **outSn) // OUT created session object w/ fields updated respectively

struct st21_session {

st21_sn_flags_t caps;//IN capabilities of the session (if single or dual path)

uint32_t ssid;	   //IN determines dynamic ID of the session

uint16_t nicPort[2];// derived from device NIC ports, second valid if dual path is supported

uint32_t timeslot;// assigned timeslot ID [0- 1st timeslot, N] from device timeslots

uint32_t trOffset;// offset of the timeslot since even EPOCH (calculated by default)

uint32_t tprs;    // time in nanoseconds of 2 consecutive packets begins of the same session

uint32_t pktTime; // time in nanoseconds of the packet

uint32_t frameSize;

uint64_t pktsDrop[ST_PKT_DROP_MAX];

uint64_t frmsDrop[ST_FRM_DROP_MAX];

uint64_t pktsLost[ST_PKT_LOST_MAX];

uint64_t pktsSend;

uint64_t frmsSend;

uint64_t pktsRecv;

uint64_t frmsRecv;

};

struct st21_format {

st21_pix_fmt_t pixelFmt;

st21_vscan_t   vscan;

uint32_t height;

uint32_t width;

uint32_t totalLines;   // 750 for 720pHD, 1125 for HD, 2250 for UHD

uint32_t trOffsetLines;// 22 for interlaced HD, 43 for progressive HD

uint32_t pixelGrpSize;// 3 for RGB, 5 for 422 10 bit - shall match the format - for sanity check

uint32_t pixelsInGrp; // number of pixels in each pixel group, 2 for 422-10

uint32_t clockRate;   //90k of sampling clock rate

uint32_t frmRateMul;  //60000 or 30000

uint32_t frmRateDen;  //1001 or 1000

st21_pkt_fmt_t pktFmt;// if single, dual or more lines of RFC4175 or other format

uint32_t pixelsInPkt; // number of pixels in each line

uint32_t pktsInLine;  // number of packet per each line

uint32_t pktSize;     //pkt size w/o VLAN header

uint32_t frameTime;   // time in nanoseconds of the frame

uint32_t pktsInFrame; //packets in frame

};

Description:

The function is called to create a ST2110-21 RTP raw video session (complying to RFC 4175) on a transmit of the receive device (the device direction determines the direction of the created session). The video type is determined by the format structure, i.e. interlaced or progressive scanning, resolution and frame rate. The created session frame rate shall be compatible with the device setting according to the table below:

| device rate (exactRate) |  | 25 | 50 | 29 | 59 |
| --- | --- | --- | --- | --- | --- |
| session rate progressive | 25 | Y | N | N | N |
| session rate progressive | 50 | Y | Y | N | N |
| session rate progressive | 29 | N | N | Y | N |
| session rate progressive | 59 | N | N | Y | Y |
| session rate interlaced | 25 | Y | N | N | N |
| session rate interlaced | 50 | Y | N | N | N |
| session rate interlaced | 29 | N | N | Y | N |
| session rate interlaced | 59 | N | N | Y | N |

The meaning of the other function parameters is as follows:

- inSn structure is to can be used to set session capabilities and ssid.

- outSn pointer shall be used to return the created session handle that is to be used in the following API calls upon successful completion.

Remarks:

Supported since prototype SW release.

Return codes and errors:

ST_OK

ST_SN_ERR_NO_TIMESLOT

ST_SN_ERR_RATE_NO_FIT

ST_FMT_ERR_BAD_PIX_FMT : ST_FMT_ERR_LAST

### 3.2 St21BindIpAdrr

Notation:

st_status_t

St21BindIpAddr(

st21_session_t *sn, // IN referred session

st_addr_t *addr,    // IN addr structure (union for IPV4 and IPV6)

uint16_t nicPort);  // IN nic port on which that address is valid (one of session NIC ports)

typedef enum st_addr_opt {

ST_ADDR_UCAST_IPV4 = 0x1,

ST_ADDR_MCAST_IPV4 = 0x2,

ST_ADDR_UCAST_IPV6 = 0x4,

ST_ADDR_MCAST_IPV6 = 0x8,

ST_ADDR_VLAN_TAG = 0x10,

ST_ADDR_VLAN_DEI = 0x20,

ST_ADDR_VLAN_PCP = 0x40,

ST_ADDR_IP_ECN   = 0x100,

ST_ADDR_IP_DSCP  = 0x200,

} st_addr_opt_t;

/**

* Structure for connection address of IPv4/v6 and UDP ports

*/

struct st_addr {

st_addr_opt_t options; // IN options selecting union fields to be used

union {

struct sockaddr_in  addr4;

struct sockaddr_in6 addr6;

} src;

union {

struct sockaddr_in  addr4;

struct sockaddr_in6 addr6;

} dst;

union {

struct {

uint16_t tag : 12; //IN VLAN tag if

uint16_t dei : 1;

uint16_t pcp : 3; // IN referred session

};

uint16_t vlan;

};

union {

struct {

uint8_t ecn  : 2;

uint8_t dscp : 6;

};

uint8_t tos;

};

};

Description:

The function is called to set destination and source IP addresses, VLAN tag, and DiffServ priority of the streams through IP dscp or VLAN pcp.  The remaining fields are expected to be 0.

This function in case of setting unicast IP address on a session belonging to a sender device is expected to send ARP request on the specific port. In case of a session created on a receiver device by setting Multicast IP address as destination address, IGMP message of a Add Member to the group selected with that address is send, so that switches listening to IGMP with IGMP Snooping can learn this IP Multicast group on the receiver switch port.

Remarks:

Supported since prototype SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### St21ListenSession

Notation:

st_status_t

St21ListenSession(

st21_session_t *sn, // IN referred session

st_addr_t *addr);   // IN addr structure (union for IPV4 and IPV6

Description:

The function is called formulticast addresses only (ST_ADDR_MCAST_IPV4 shall be specified in options field of the st_addr_t structure) on the producer side to initiate sending IGMP Queries of the multicast group so that the receivers can respond to these queries with IGMP Member Report packets. The IGMP group address is extracted from the dst field of the st_addr_t structure while the other structure members are ignored.

Remarks:

Supported since Beta SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### St21JoinSession

Notation:

st_status_t

St21JoinSession(

st21_session_t *sn, // IN referred session

st_addr_t *addr);   // IN addr structure (union for IPV4 and IPV6)

Description:

The function is called for the multicast addresses only (ST_ADDR_MCAST_IPV4 is specified in options field of the st_addr_t structure) on the consumer side to initiate responding to IGMP Queries of the multicast group with IGMP Member Report packets. The IGMP group address is extracted from the dst field of the st_addr_t structure while the other structure members are ignored. Thanks to these responses, the network switch implementing IGMP Snooping can learn the IGMP MAC addresses of the groups to which there are active multicast receivers.  This allows the multicast packets to be passed to these enabled switch ports.

Remarks:

Supported since Beta SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### St21DropSession

Notation:

st_status_t

St21DropSession(

st21_session_t *sn); // IN referred session

Description:

The function is called for stopping IGMP activities, both on the producer (transmitter) side that is sending IGMP Queries of the multicast group and on the consumer (receiver) side which is responding to the IGMP queries with IGMP Member Report packets.

Remarks:

Supported since Beta SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### St21GetFormat

Notation:

st_status_t

St21GetFormat(

st21_session_t *sn,  // IN referred session

st21_format_t *fmt); // OUT format

Description:

The function is called to retrieve the format information of the existing session.

Remarks:

Supported since prototype SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### St21SetParam

Notation:

st_status_t

St21SetParam(

st21_session_t *sn, // IN referred session

st_param_t prm, // IN parameter ID

uint32_t val);  // IN value

Description:

The function is called setting session specific parameters for example: enable/disable redundancy path, enable/disable slice level notifications, etc.

Remarks:

Supported since Beta SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### St21GetParam

Notation:

st_status_t

St21SetParam(

st21_session_t *sn, // IN referred session

st_param_t prm,  // IN parameter ID

uint32_t *val);  // OUT value

Description:

The function is called for getting session specific parameters for example: state of enable/disable of the redundancy path, state of enable/disable of the slice level notifications, etc.

Remarks:

Supported since Beta SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

## St21 Producer API group

### St21RegisterProducer

Notation:

st_status_t

St21RegisterProducer (

st21_session_t *sn,   // IN session pointer

st21_producer_t *prod)// IN producer structure to register

struct st21_producer

{

void            *appHandle;

st21_prod_type_t prodType; // slice mode or frame mode w/ or w/o timestamp

uint32_t         frameSize;

uint32_t         sliceSize;

uint32_t         sliceCount; // count of slices in Frame if slice mode

St21GetNextFrameBuf_f    St21GetNextFrameBuf;

St21GetNextSliceOffset_f St21GetNextSliceOffset;

St21GetFrameTmstamp_f    St21GetFrameTmstamp;

St21NotifyFrameDone_f    St21NotifyFrameDone;

St21NotifySliceDone_f    St21NotifySliceDone;

St21NotifyStopDone_f     St21NotifyStopDone;

St21BuildUdpPkt_f        St21BuildUdpPkt;

};

typedef struct st21_producer st21_producer_t;

typedef enum st21_prod_type {

ST21_PROD_INVALID,

ST21_PROD_P_FRAME,//producer of complete frames in progressive mode

ST21_PROD_P_FRAME_SLICE,//producer of slices of progressive frames

ST21_PROD_P_FRAME_TMSTAMP,//producer of complete frames and having SDI timestamp already determined

ST21_PROD_P_SLICE_TMSTAMP,//producer of sliced frames and having SDI timestamp already determined at the frame level

ST21_PROD_I_FIELD,//producer of interlaced fields

ST21_PROD_I_FIELD_SLICE,//producer of slices of interlaced fields

ST21_PROD_I_FIELD_TMSTAMP,//producer of interlaced fields and having SDI timestamp already determined

ST21_PROD_I_SLICE_TMSTAMP,//producer of slices of interlaced fields and having SDI timestamp already determined at the field level

ST21_PROD_RAW_UDP,//producer that assembles own UDP frames, it uses only callback of St21BuildUdpPkt_f

} st21_prod_type_t;

Description:

The function is called to register a live video producer for video streaming within the ST2110-21 RTP raw video session. The producer should implement callbacks depending on its type as described by the below table. Notice that for ST21_PROD_RAW_UDP only callbacks of St21BuildUdpPkt_f should implemented.

| Function | ST21_PROD_P_FRAME ST21_PROD_I_FIELD | ST21_PROD_P_FRAME_SLICE ST21_PROD_I_FIELD_SLICE | ST21_PROD_P_FRAME_TMSTAMP ST21_PROD_I_FIELD_TMSTAMP | ST21_PROD_P_FRAME_SLICE_TMSTAMP ST21_PROD_I_FIELD_SLICE_TMSTAMP |
| --- | --- | --- | --- | --- |
| St21GetNextFrameBuf_f | Required | Required | Required | Required |
| St21GetNextSliceOffset_f | n/a | Required | n/a | Required |
| St21GetFrameTmstamp_f | n/a | n/a | n/a | Required |
| St21NotifyFrameDone_f | Required | Required | Required | Required |
| St21NotifySliceDone_f | n/a | Required | n/a | Required |
| St21NotifyStopDone_f | Required | Required | Required | Required |

The meanings of the producer types are the following:

| Type | Description | Support since |
| --- | --- | --- |
| ST21_PROD_P_FRAME | Basic producer w/ the frame buffer granularity level of complete frames in progressive mode | Prototype |
| ST21_PROD_I_FIELD | Basic producer w/ the interlaced field buffer granularity level of complete fields in interlaced mode | Prototype |
| ST21_PROD_P_FRAME_SLICE | Producer as the frame buffer granularity level of slices of the frames in progressive mode. Slices must be complete lines and continuous fragments of the frame and their practical number shall be limited due to performance reasons to not more than several dozens. | Beta |
| ST21_PROD_I_FIELD_SLICE | Producer as the interlaced field granularity level of complete fields in interlaced mode. Slices must be complete lines and continuous fragments of the field and their practical number shall be limited due to performance reasons to not more than several dozens. | Beta |
| ST21_PROD_P_FRAME_TMSTAMP | Basic producer w/ the frame buffer granularity level of complete frames in progressive mode that has timestamp determined by the producer externally to the library | Beta |
| ST21_PROD_I_FIELD_TMSTAMP | Basic producer w/ the interlaced field buffer granularity level of complete fields in interlaced mode that has timestamp determined by the producer externally to the library | Beta |
| ST21_PROD_P_SLICE_TMSTAMP | Producer as the frame buffer granularity level of slices of the frames in progressive mode. Slices must be complete lines and continuous fragments of the frame and their practical number shall be limited due to performance reasons to not more than several dozens. | Beta |
| ST21_PROD_I_SLICE_TMSTAMP | Producer as the interlaced field granularity level of complete fields in interlaced mode. Slices must be complete lines and continuous fragments of the field and their practical number shall be limited due to performance reasons to not more than several dozens. | Beta |
| ST21_PROD_RAW_UDP | Producer that assembles by their own UDP frames, it uses only callback of St21BuildUdpPkt_f | Beta |

Other parameters have the following meaning:

- appHandle handle passed back to the producer application upon each callback

- frameSize size of the frame shall match the session setting

- sliceSize size of the slice for the producer (determines session settings)

- sliceCount number of slices for the producer (determines session settings)

Remarks:

Supported since prototype SW release with basic frame/field granularity.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

ST_SN_ERR_NO_TIMESLOT

ST_BAD_PRODUCER

ST_PROD_ALREADY_REGISTERED

### St21ProducerStartFrame

Notation:

st_status_t

St21ProducerStartFrame (

st21_session_t *sn, // IN referred session

uint8_t        *frameBuf,  //IN 1st frame buffer for the session

uint32_t       linesOffset,//IN offset in complete lines of the frameBuf to which producer filled the buffer

uint32_t       tmstamp,    //IN if not 0 then 90kHz timestamp of the frame

uint64_t       ptpTime);   //IN if not 0 start new frame at the given PTP timestamp

Description:

The function is called for start streaming of the producer session. If both tmstamp and ptpTime are specified then the producer session starts at those selected moments (which might be in the future as specified by ptpTime or within the current frame epoch if ptpTime to epoch difference does not exceeds TRoffset specified for that session.

Remarks:

Supported since Beta SW release.

Return codes and error:

ST_OK

ST_INVALID_PARAM

### St21ProducerUpdate

Notation:

st_status_t

St21ProducerUpdate(

st21_session_t *sn, // IN referred session

uint8_t        *frameBuf,  //IN 1st frame buffer for the session

uint32_t       linesOffset)//IN offset in complete lines of the frameBuf to which producer filled the buffer

Description:

The function is called for restartting video streaming of the producer session after the session was on hold due to a slow producer.

Remarks:

Supported since Beta SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### St21ProducerStop

Notation:

st_status_t

St21ProducerStop (

st21_session_t *sn) // IN referred session

Description:

The function is called for stopping video streaming of the producer session.

Remarks:

Supported since Beta SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

## 4. St21 Consumer API group

### St21RegisterConsumer

Notation:

st_status_t

St21RegisterConsumer (

st21_session_t *sn,   // IN session pointer

st21_consumer_t *cons)// IN consumer structure to register

struct st21_producer

{

void            *appHandle;

st21_cons_type_t consType; // slice mode or frame mode w/ or w/o timestamp

uint32_t         frameSize;

uint32_t         sliceSize;

uint32_t         sliceCount; // count of slices in Frame if slice mode

St21GetNextFrameBuf_f    St21GetNextFrameBuf;

St21GetNextSliceOffset_f St21GetNextSliceOffset;

St21NotifyFrameRecv_f    St21NotifyFrameRecv;

St21NotifyFrameSlice_f   St21NotifySliceRecv;

St21PutFrameTmstamp_f    St21PutFrameTmstamp;

St21NotifyFrameDone_f    St21NotifyFrameDone;

St21NotifySliceDone_f    St21NotifySliceDone;

St21NotifyStopDone_f     St21NotifyStopDone;

St21RecvUdpPkt_f         St21RecvUdpPkt;

};

typedef struct st21_consumer st21_consumer_t;

typedef enum st21_cons_type {

ST21_CONS_INVALID,

ST21_CONS_P_FRAME,//consumer of complete frames in progressive mode

ST21_CONS_P_FRAME_SLICE,//consumer of slices of progressive frames

ST21_CONS_P_FRAME_TMSTAMP,//consumer of complete frames and reusing RTP timestamp as SDI timestamp

ST21_CONS_P_SLICE_TMSTAMP,//consumer of sliced frames and reusing RTP timestamp as SDI timestamp

ST21_CONS_I_FIELD,//consumer of interlaced fields

ST21_CONS_I_FIELD_SLICE,//consumer of slices of interlaced fields

ST21_CONS_I_FIELD_TMSTAMP,//consumer of interlaced fields and reusing RTP timestamp as SDI timestamp

ST21_CONS_I_SLICE_TMSTAMP,//consumer of slices of interlaced fields and reusing RTP timestamp as SDI timestamp

ST21_CONS_RAW_UDP,//consumer that assembles own UDP frames, it uses only callback of St21RecvUdpPkt_f

} st21_cons_type_t;

Description:

The function is called to register a live video consumer for video streaming within the ST2110-21 RTP raw video session. The consumer should implement callbacks depending on its type as described by the below table. Notice that for ST21_CONS_RAW_UDP only callbacks of St21RecvUdpPkt_f shall be implemented.

| Function | ST21_CONS_P_FRAME ST21_CONS_I_FIELD | ST21_CONS_P_FRAME_SLICE ST21_CONS_I_FIELD_SLICE | ST21_CONS_P_FRAME_TMSTAMP ST21_CONS_I_FIELD_TMSTAMP | ST21_CONS_P_FRAME_SLICE_TMSTAMP ST21_CONS_I_FIELD_SLICE_TMSTAMP |
| --- | --- | --- | --- | --- |
| St21GetNextFrameBuf_f | Required | Required | Required | Required |
| St21GetNextSliceOffset_f | n/a | Required | n/a | Required |
| St21NotifyFrameRecv_f | Required | Required | Required | Required |
| St21NotifySliceRecv_f | n/a | Required | n/a | Required |
| St21PutFrameTmstamp_f | n/a | n/a | n/a | Required |
| St21NotifyFrameDone_f | Required | Required | Required | Required |
| St21NotifySliceDone_f | n/a | Required | n/a | Required |
| St21NotifyStopDone_f | Required | Required | Required | Required |

The meanings of the producer types are the following:

| Type | Description | Support since |
| --- | --- | --- |
| ST21_CONS_P_FRAME | Basic consumer w/ the frame buffer granularity level of complete frames in progressive mode | Prototype |
| ST21_CONS_I_FIELD | Basic consumer w/ the interlaced field buffer granularity level of complete fields in interlaced mode | Prototype |
| ST21_CONS_P_FRAME_SLICE | Consumer as the frame buffer granularity level of slices of the frames in progressive mode. Slices must be complete lines and continuous fragments of the frame and their practical number shall be limited due to performance reasons to not more than several dozens. | Beta |
| ST21_CONS_I_FIELD_SLICE | Consumer as the interlaced field granularity level of complete fields in interlaced mode. Slices must be complete lines and continuous fragments of the field and their practical number shall be limited due to performance reasons to not more than several dozens. | Beta |
| ST21_CONS_P_FRAME_TMSTAMP | Basic consumer w/ the frame buffer granularity level of complete frames in progressive mode that has timestamp determined by the consumer externally to the library | Beta |
| ST21_CONS_I_FIELD_TMSTAMP | Basic consumer w/ the interlaced field buffer granularity level of complete fields in interlaced mode that has timestamp determined by the consumer externally to the library | Beta |
| ST21_CONS_P_SLICE_TMSTAMP | Consumer as the frame buffer granularity level of slices of the frames in progressive mode. Slices must be complete lines and continuous fragments of the frame and their practical number shall be limited due to performance reasons to not more than several dozens. | Beta |
| ST21_CONS_I_SLICE_TMSTAMP | Consumer as the interlaced field granularity level of complete fields in interlaced mode. Slices must be complete lines and continuous fragments of the field and their practical number shall be limited due to performance reasons to not more than several dozens. | Beta |
| ST21_CONS_RAW_UDP | Consumer that assembles by their own UDP frames, it uses only callback of St21RecvUdpPkt_f | Beta |

Other parameters have the following meaning:

- appHandle handle passed back to the consumer application upon each callback

- frameSize size of the frame shall match the session setting

- sliceSize size of the slice for the consumer (determines session settings)

- sliceCount number of slices for the consumer (determines session settings)

Remarks:

Supported since Prototype SW release with basic frame/field granularity.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

ST_SN_ERR_NO_TIMESLOT

ST_BAD_CONSUMER

ST_CONS_ALREADY_REGISTERED

### St21ConsumerStartFrame

Notation:

st_status_t

St21ConsumerStartFrame (

st21_session_t *sn, // IN referred session

uint8_t        *frameBuf,  //IN 1st frame buffer for the session

uint32_t       linesOffset,//IN offset in complete lines of the frameBuf to which consumer filled the buffer

uint32_t       tmstamp,    //IN if not 0 then 90kHz timestamp of the frame

uint64_t       ptpTime);   //IN if not 0 start new frame at the given PTP timestamp

Description:

The function is called to start receivingthe consumer session. If both of tmstamp and ptpTime are specified then the consumer session starts at those selected moments (which might be in the future as specified by ptpTime or within the current frame epoch if ptpTime to epoch difference does not exceeds TRoffset specified for that session.

Remarks:

Supported since Beta SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### St21ConsumerUpdate

Notation:

st_status_t

St21ConsumerUpdate(

st21_session_t *sn, // IN referred session

uint8_t        *frameBuf,  //IN 1st frame buffer for the session

uint32_t       linesOffset)//IN offset in complete lines of the frameBuf to which consumer can fill the buffer

Description:

The function is called to restart video streaming the consumer session after the session was on hold due to a slow consumer.

Remarks:

Supported since Beta SW release.

Return codes and errors:

ST_OK

ST_INVALID_PARAM

### St21ConsumerStop

Notation:

st_status_t

St21ConsumerStop (

st21_session_t *sn) // IN referred session

Description:

The function is called to stop video streaming of the consumer session.

Remarks:

Supported since Beta SW release.

Return codes and errors (some explained):

ST_OK

ST_INVALID_PARAM

# Using Software

Recommended OS - Ubuntu 18.04 LTS with kernel 5.3

1. Edit /etc/default/grub file

vim /etc/default/grub

2. Change following line

GRUB_CMDLINE_LINUX_DEFAULT="splash default_hugepagesz=1G hugepagesz=1G hugepages=8 nomodeset"

3. Edit /etc/fstab file

vim /etc/fstab

4. Add following line

nodev	/mnt/huge_1GB	hugetlbfs	pagesize=1GB 0	0

5. Update your software:

Sudo apt update

Sudo apt upgrade

6. Install software:

sudo add-apt-repository "deb http://us.archive.ubuntu.com/ubuntu/ saucy universe multiverse"

sudo add-apt-repository "deb http://us.archive.ubuntu.com/ubuntu/ saucy-updates universe multiverse"

apt install -y dpdk-igb-uio-dkms gcc cmake python python3 python3-pip libnuma-dev pkg-config libsdl2*-dev

pip3 install meson ninja

7. Enable UIO driver:

modprobe uio

8. Configure DPDK:

8.1 Preparation of sources

8.1.1 Using latest stable version of DPDK library from GitHub (preferred)

Clone DPDK repo

git clone https://github.com/DPDK/dpdk.git <NameOfBaseDir>

Checkout to v20.08 version

cd <NameOfBaseDir>

git checkout --no-track -b v20.08 v20.08

8.1.2 Using internal copy on Intel GitLab (not use it)

git clone -b dpdk ssh://git@gitlab.devtools.intel.com:29418/VEI/dpdk_media_streamer.git <NameOfBaseDir>

pushd <NameOfBaseDir>

git remote add dpdk.org https://dpdk.org/git/dpdk

git remote add github.com https://github.com/DPDK/dpdk.git

8.2 Configure library features

8.2.1 Set meson option in meson_options.txt

enable_kmods => true use_hpet => true

8.2.2 Add new build options in /meson_options.txt

option('support_ieee1588', type: 'boolean', value: true, description: 'support IEEE1588(PTP) ')

8.2.3 Add configuration to /config/meson.build around line 230 (preferred)

dpdk_conf.set('RTE_LIBRTE_IEEE1588', get_option('support_ieee1588'))

8.3 Build and install

meson <NameOfOutDir>

cd <NameOfOutDir>

ninja

sudo ninja install

sudo ldconfig

pkg-config --cflags libdpdk

pkg-config --libs libdpdk

cd ..

8.4 Remarks

If you want to uninstall DPDK library safely use command:

cd  <NameOfOutDir>

ninja uninstall

# Build and run

### Build with installation

1. Get resources

Clone Intel® Media Streaming Library for ST 2110 repo

git clone https://gitlab.devtools.intel.com/VEI/dpdk_media_streamer.git
 cd dpdk_media_streamer

2. Build library

cd <MediaStreamerDir>

meson <NameOfLibOutDir>

cd <NameOfLibOutDir>

ninja

ninja install

3. Build applications

cd <MediaStreamerDir>/app

meson <NameOfAppOutDir>

cd <NameOfAppOutDir>

ninja

### Build without installation

1. Get resources

Clone Intel® Media Streaming Library for ST 2110 repo

git clone https://gitlab.devtools.intel.com/VEI/dpdk_media_streamer.git
 cd dpdk_media_streamer

2. Build library

cd <MediaStreamerDir>

meson <NameOfLibOutDir>

cd <NameOfLibOutDir>

ninja

3. Build applications

cd <MediaStreamerDir>/app

meson <NameOfAppOutDir> -Duselocalmslib=true

cd <NameOfAppOutDir>

ninja

4. Run

${NameOfAppOutDir}/rx_app/RxApp --in_port <input PCI device address> --mac <destination MAC address> --ip <destination IP address> --sip <source IP address> --o_port <output PCI device address> --

Available parameters:

-h                                           : print help info (optional)

-v                                           : print versions info (optional)

--mac <MAC addr>             : destination MAC address (required)

--ip <IP addr>                       : destination IP address (required)

--sip <IP addr>                     : source IP address (required)

--mip                                      : enable multicast flag

--ebu                                      : enable EBU compatibility logs

--port <UDP port>               : base port from which to iterate sessions port IDs (optional)

--rx                                         : run receive mode only (optional)

--tx                                         : run transmit mode only (optional)

--rgba                                     : input / output buffers are in rgba format

--yuv10be                              : input / output buffers are in yuv10be format

--format <fmt string>          : select frame format e.g. a1080i50 = aya 1080 interlaced 50fps (optional)

: e.g. i720p29  = intel 720 progressive 29.97fps

: e.g. i1080p59 = intel 1080 progressive 59.94fps

: e.g. i2160p59 = intel 2160 progressive 59.94fps

: e.g. i1080i29 = intel 1080 interlaced 29.97fps

: e.g. a1080p59 = aya 1080 progressive 59.94fps

--s_count <number of sessions>	             : number of sessions (optional)

--o_port <PCI device address>	                 : output interface PCI device address (required)

--in_port <PCI device address>	                : input interface PCI device address (required)

--ptp <hhhhhh.hhhh.hhhhhh>                 : master clock id

--log_level <user,level<info/debug/error>>   : enable additional logs

4. Running application using environment variables

Edit /etc/environment file

vim /etc/environment

Set following variables

RTE_SDK=/home/dpdk

RTE_TARGET=x86_64-native-linux-gcc

IN_PORT=0000:af:00.1                     # PCI addres of NIC to recv frames

OUT_PORT=0000:af:00.0                 # PCI adress of NIC to send frames

OUT_IP=192.168.0.2                         # IP addres for sender

DEST_IP=192.168.0.1                        # IP addres of destination

DEST_MAC=12:34:56:ab:cd:ef         # MAC of destination

ST_NIC_RATE_GBPS=ST_NIC_RATE_SPEED_10GBPS

ST_RX='--rx'

${NameOfAppOutDir}/rx_app/RxApp ${ST_RX-} --in_port $IN_PORT --mac $DEST_MAC --ip $DEST_IP --sip $OUT_IP --o_port $OUT_PORT --

# Appendix A

Error codes

| ST_OK | upon successful completion |
| --- | --- |
| ST_INVALID_PARAM | upon using invalid value in one of inDev structure parameters |
| ST_OUT_OF_MEMORY | generated if the report buffer cannot be created due to not enough memory available, and the location pointed by queryHandle returns NULL |
| ST_GENERAL_ERROR | upon a critical system condition (indicates memory corruption) |
| ST_SN_ERR_NO_TIMESLOT | upon lack of available timeslots for the sessions on device |
| ST_SN_ERR_RATE_NO_FIT | upon incompatibility of frame rate of the created session and the one set for the device |
| ST_FMT_ERR_BAD_PIX_FMT : ST_FMT_ERR_LAST | error from this range indicates specific format error |
| ST_BAD_PRODUCER | upon incompatibility of frame size or slice size / count |
| ST_BAD_CONSUMER | upon incompatibility of frame size or slice size / count |
| ST_CONS_ALREADY_REGISTERED | if the consumer is already present on the session |
| ST_PROD_ALREADY_REGISTERED | if the producer is already present on the session |